Questionnaire for Nutsche Filter-Dryer

Filtration Phase :

- What is the material we want to filter-dry ? Please give us a full detailed explanation of the different ingredients ( % of materials, Bulk Densities, Viscosities, etc )

- What is the particle size of the solids we need to filter ?

- What is the shape of the solids ? ( Crystals/pearls/amorphic )

- Contact and non-contact parts material of construction ? Finish ?

- What is the size of batch to be filtered ( in Lit. ) ? Just check again the % of solids remain from that batch.

- Did they use a Nutsche filter today ? If yes, please gather ALL the information about it :

  - What is the max. cake height possible ?

  - What is the BD of the wet cake ? ( after filtering – before drying )

  - What is the liquids/solids content in the cake ?

  - Did they use sintered mesh or cloth ?

  - What is the filtering working pressure ?

  - What is good/bad with the existing Nutsche ?

- If they don't do it today, we need to know if the already have an idea about the above questions ? Did they made a test already ? Gather all the info. About the test.

- What is the expected time for this process ?

- Does the filter needs to be ExP ?

Washing Phase :

- Is washing done ?

- How ? Please give a full and accurate explanation about the washing.

- Does it needs to be re slurry ? Or just top-to-bottom wash is enough ?

- At the end of process, does you CIP/SIP the filter ?

- What is the expected time for this process ?

Drying Phase :

- What is the liquid to be dry ? Please give us all the information about it : Viscosity, Bulk Density, Heat capacity, Boiling Temp.,etc.

- What is the required final moisture content ? Just check again the amount of liquids needed to be evaporated from that batch.

- What will be the expected drying temp. in the filter ?

- What will be the heating media ?

- What is the needed working pressure of the jacket ?

- What is the needed working vacuum of the filter-dryer ?

- Does the material tends to create dust when dried ?

- What is the BD of the dried solids ?

- Are the dry solids free flowing ?